SIMATS ENGINEERING

## ASSESSMENT TOOL 1

COURSE NAME:	CSA07 COURSE CODE: Computer Networks

COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks. (BL2, BL3)

Assessment Tool Weightage: 40%

Annexure A

SHORT ANSWER TEST

Code of Conduct:

I C.Manoj Kumar(192472008)	(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Annexure A

Short Answer Test

- A company needs to transfer a 150 MB design file from a client computer to a remote server over a network with a bandwidth of 50 Mbps. The Transport Layer divides the file into 1500-byte segments, while the Data Link Layer encapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

- A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

- A 12,000-byte IP packet is transmitted through a router where the Maximum Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20-byte IP header, calculate the total number of fragments created and the total header overhead introduced. Explain how the Network Layer ensures successful delivery of fragmented packets to the destination.

- A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

- A multimedia application transfers a 600 MB video file. The Session Layer inserts a synchronization checkpoint after every 60 MB of transmitted data. During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the amount of data that must be retransmitted after recovery. Explain the importance of synchronization in the Session Layer.

- Before transmission, the Presentation Layer compresses a 900 MB file by 40%. The compressed data is encrypted, adding 5% overhead to the compressed size. Calculate the compressed file size, the encrypted file size, and the total percentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

- An FTP application transfers 3 GB of data over a network with an effective throughput of 120 Mbps. The Application Layer divides the data into requests of 3 MB each. Calculate the total number of requests generated and estimate the total transmission time in seconds. Discuss how the Application Layer provides reliable services to users.

- A packet experiences the following processing delays at different OSI layers: Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, and the transmission delay is 12 ms. Calculate the total end-to-end delay experienced by the packet and explain how each layer contributes to the communication process.

- A network transfers 80 MB of useful data using frames of 1600 bytes, where each frame contains 80 bytes of protocol overhead added by different OSI layers.

Calculate the total number of frames required, the total overhead transmitted, and the transmission efficiency as a percentage. Explain how protocol overhead affects network performance.

- A hospital transfers a patient's medical record of 240 MB through an IoT-enabled healthcare network. The Presentation Layer compresses the data by 30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the compressed file size, the total number of segments transmitted, the total protocol overhead introduced, and the final amount of data transmitted across the Physical Layer. Explain how multiple OSI layers work together to ensure secure and reliable communication.

RUBRICS FOR CALCULATION

| Criteria | Weightag e | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understandin g of Concepts | 25% | Demonstrates a thorough, accurate understandin g of concepts | Good understanding with minor gaps | Basic understanding with some misconception s | Poor understanding; major misconception s |
| Explanation | 25% | Detailed, well-explained answers with relevant examples | Clear explanation with adequate details | Limited explanation; lacks depth | Poor or unclear explanation |
| Application | 20% | Effectively applies concepts with strong analytical ability | Applies concepts with minor errors | Limited application with weak analysis | No application or incorrect analysis |
| Organization | 15% | Well-structured answers with logical flow | Organized with minor issues in flow | Basic structure, lacks clarity | Poor organization, incoherent answers |

|  |  | and coherence |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Accuracy | 10% | Completely accurate and covers all required points | Mostly accurate with minor omissions | Partially correct with missing points | Incorrect answers with major gaps |
| Clarity | 5% | Clear, neat, professional presentation | Understandabl e with minor issues | Limited clarity, readability issues | Poorly written, difficult to understand |

# Design File Transfer

## Given:

  - File size = 150 MB

  - Bandwidth = 50 Mbps

  - Segment size = 1500 bytes

  - Frame overhead = 40 bytes Calculation

## Total segments

150 × 1024 × 1024 = 157,286,400 bytes

157,286,400

Frames transmitted

= 104,858 frames

Total Data Link overhead

1500

= 104,858 segments

104,858 × 40 = 4,194,320 bytes

≈ 4 MB

## Transmission time

150 MB = 1200 Mb

## Answer

  - Segments = 104,858

  - Frames = 104,858

  - Overhead = 4,194,320 bytes

1200

50

= 24 seconds

  - Transmission time = 24 seconds Explanation

  - Transport Layer divides the file into segments and provides reliable delivery using acknowledgements and retransmission.

  - Data Link Layer adds headers/trailers, detects errors using CRC, and delivers frames correctly over the physical link.

# Sliding Window

## Given

  - Window size = 6

  - Total frames = 48

## Calculation

Windows required

48/6 = 8

One frame lost in each window Retransmissions = 8

## Answer

  - Windows required = 8

  - Retransmissions = 8 frames Flow Control

Flow control prevents the sender from sending data faster than the receiver can process,

reducing congestion and improving efficiency.

# IP Fragmentation

## Given

Packet size = 12,000 bytes MTU = 1500 bytes

IP Header = 20 bytes Calculation

Payload per fragment

1500 − 20 = 1480 bytes

Fragments

12000/1480 = 8.1

Therefore, Fragments = 9 Header overhead

9 × 20 = 180 bytes

## Answer

- Fragments = 9

- Header overhead = 180 bytes Explanation

The Network Layer fragments packets according to the MTU and reassembles them correctly at the destination using fragment offset and identification fields.

# Sliding Window

Same as Question 2.

## Answer

  - Windows = 8

  - Retransmissions = 8

Flow control improves communication by avoiding buffer overflow and maintaining efficient data transfer.

# Session Layer

## Given

Video = 600 MB Checkpoint every 60 MB Failure after 390 MB Calculation

Checkpoints before failure 60,120,180,240,300,360

## = 6 checkpoints

Last checkpoint = 360 MB Retransmission

390 − 360 = 30𝑀𝐵

## Answer

- Checkpoints = 6

- Retransmitted data = 30 MB Explanation

Synchronization checkpoints allow transmission to resume from the last checkpoint instead of restarting from the beginning.

# Presentation Layer

## Given

Original file = 900 MB Compression = 40% Encryption overhead = 5% Calculation

Compressed size

900 × 0.60 = 540𝑀𝐵

Encrypted size

540 × 1.05 = 567𝑀𝐵

Reduction

900 − 567 = 333𝑀𝐵

Percentage reduction

## Answer

- Compressed size = 540 MB

- Encrypted size = 567 MB

- Reduction = 37% Explanation

333

900

× 100 = 37%

Presentation Layer compresses data to reduce transmission size and encrypts data to improve security.

# FTP Transfer

## Given

Data = 3 GB Throughput = 120 Mbps Request size = 3 MB Calculation

3 GB = 3072 MB

Requests

3072/3 = 1024

Transmission time 3 GB = 24,576 Mb

24576/120 = 204.8 s

## Answer

- Requests = 1024

- Transmission time = 204.8 seconds Explanation

The Application Layer provides services like FTP for reliable file transfer, user authentication, and error handling.

# End-to-End Delay

Processing delays Application = 4 ms Transport = 3 ms Network = 5 ms Data Link = 2 ms Physical = 1 ms Propagation = 18 ms

Transmission = 12 ms

## Calculation

4 + 3 + 5 + 2 + 1 + 18 + 12 = 45𝑚𝑠

## Answer

Total end-to-end delay = 45 ms Explanation

Each OSI layer performs specific functions such as data formatting, routing, framing, and transmission, contributing to the total delay.

# Frame Overhead

## Given

Useful data = 80 MB Frame size = 1600 bytes Overhead = 80 bytes Payload

1600 − 80 = 1520 bytes

Useful data

80 × 1024 × 1024 = 83,886,080 bytes

Frames

83886080/1520 = 55189

Overhead

55189 × 80 = 4,415,120 bytes

Efficiency

## Answer

- Frames = 55,189

1520

1600

× 100 = 95%

- Total overhead = 4,415,120 bytes

- Efficiency = 95% Explanation

Protocol overhead consumes bandwidth, reducing transmission efficiency, although it is necessary for addressing, error detection, and reliable communication.

# Hospital IoT Network

## Given

Original file = 240 MB Compression = 30% Segment size = 1400 bytes Frame overhead = 60 bytes Calculation

Compressed file

240 × 0.70 = 168𝑀𝐵

Compressed bytes

168 × 1024 × 1024 = 176,160,768

Segments

176160768/1400 = 125830

Overhead

125830 × 60 = 7,549,800 bytes

Final transmitted data

176160768 + 7549800 = 183710568 bytes

## ≈ 175.2 MB

Answer

- Compressed file = 168 MB

- Segments = 125,830

- Protocol overhead = 7,549,800 bytes

- Final transmitted data = 183,710,568 bytes (≈175.2 MB)